# Requirements

## Control Box

- I2C interface (CAN module)

- SPI interface (LED driver)

- Large number of digital pins (2 for I2C, 2 for SPI, 1 per 8 digit display SPI CS, 1 per switch)

## Actuation Box

- I2C interface (CAN module, DAQ box)

- Digital pins for actuation (solenoids, servo)

# Microcontroller Comparison

| Microcontroller | Arduino Mega 2560 | Arduino Uno | Teensy 3.5 | Teensy 4.1 |
| --- | --- | --- | --- | --- |
| Cost | $20 | $18 | $35 but not readily available | $37 |
| CoreMark Rating | 7 | <= 7 | 266 | 2314 |
| Flash Memory | 256kB | 32kB | 512kB | 7936kB |
| Communication Interfaces | I2C, SPI, UART | I2C, SPI, UART | I2C, SPI, UART, MicroSD | I2C, SPI, UART, MicroSD |
| # Digital Pins | 54 | 14 | 35 | 35 |
| # Analog Pins | 16 | 6 | 27 | 18 |
| Max Pin Current | 20mA continuous, 40mA burst | 20mA continuous, 40mA burst | 10mA | 10mA |
| Logic Level | 5V | 5V | 3.3V IS 5V Tolerant | 3.3V NOT 5V tolerant |
| Estimated Power Draw | 270mW | 230mW | 350mW | 500mW |
| Power Options (VIN) | 5V or 7V to 12V | 5V or 7V to 12V | 3.6V - 5.5V | 3.6V - 5.5V |
| Temperature Rating | -40°C - 85°C | -40°C - 85°C | -40°C - 85°C | -40°C - 85°C |
| Other | Not solderable unless getting the board | Not solderable | MicroSD Card, solderable | MicroSD Card, solderable |

# Choice and Rationalization

The Arduino Mega 2560 looks like the best board for both the control box and the actuation box.

- The benefits of the Teensy 4.1 (extra RAM and flash memory as well as faster processor) are not required for either of the two applications. This is because there is not much computation being done and a communication rate of 2Hz would be adequate enough, which all of the boards can do quite easily.

- The Mega has the most digital pins which is the highest priority as without them you would be limited on the number of switches and displays that could be connected. 54 digital pins is more than enough and can help alleviate any future problems with lack of I/O.

- The largest drawback of the Mega besides its relatively slow speed compared to the Teensy is its large footprint. By getting the embeddable board version, this is mostly alleviated and allows soldering to a PCB or protoboard.