# Requirements

- Measure the ambient temperature to within 1°C

- Have a measuring range at least -10°C to 80°C

# Temperature Sensor Comparison

| Sensor | DHT11 | DHT22 | DS18B20 | TMP36 |
| --- | --- | --- | --- | --- |
| Cost (# included) | $10.29 (5) | $14.49 (2) | $11.58 (5) | $9.99 (5) |
| Range | 0°C - 50°C | -40°C - 125°C | -55°C - 125°C | -40°C - 150°C |
| Accuracy | ±2°C | ±0.5°C | ±0.5°C | ±2°C |
| Communication Interface | Digital | Digital | Digital, OneWire | Analog |

# Choice and Rationalization

The DS18B20 looks like the best choice.

- It covers more than the normal operating range of the microcontrollers and other components, as well as being highly accurate.

- Easy interface using a digital input.

- Cheap and readily available