# Requirements

- Transmit consistently over ~100ft

- Not wireless due to metal interference and lack of reliability

- No high data rate needed

- Works over a standard CAT5 ethernet cable or other standard wire that can be found ~100ft

- Preexisitng libraries and documentation

# Communication Protocol Comparison

| Protocol | CAN | RS232 | RS485 | I2C | Arduino’s UART |
| --- | --- | --- | --- | --- | --- |
| Module Cost (#) | $11 (2) | $7.5 (5) | $10 (5) | $0 | $0 |
| Realistic Range | 40m-1Mbps 1000m-50kbps | 15m 100m-750kbps | 1000m-60kbps | 10m | 3m |
| Standard Cable? | Yes | Yes | Yes | Yes | Yes |
| Documentation with Arduino? | Yes, lots Libraries readily available | Yes Libraries more difficult to find | Yes, good Libraries readily available | Yes, lots Libraries readily available | Yes, lots Libraries readily available |
| Offloads Processing? | Mostly Still requires SPI to get/send messages | No Similar to a LLC | No Similar to a LLC | N/A | N/A |
| Module Purchase Link | https://www.amazon.com/HiLetgo-MCP2515-TJA1050-Receiver-Arduino/dp/B01D0WSEWU/ | https://www.amazon.com/HiLetgo-MAX3232-Converter-Module-Serial/dp/B00LPK0Z9A/ | https://www.amazon.com/YWBL-WH-RS-485-Converter-Adapter-Raspberry/dp/B07S1BN6ZK/ |  |  |

# Communication Protocol Choice and Rationalization

CAN looks like the best communication protocol for the control box to actuation box link.

- Easily transmits over the required distance with a standard CAT5 ethernet cable.

- There are modules designed to work with the Arduino over SPI that are well documented with libraries available.

- I am more comfortable with CAN communication than RS485.